1、Flash Update

- How to force update the s/w when STB can not startup.

- Download the lastest s/w from forum.extract the file and get the xxx.bin

- Insert a usb stick on PC and copy the file to it.Make a dir named RomUpdate rename xxx.bin to RomUpdate.bin and move it to RomUpdate folder.insert usb stick to STB.

- Press Fontpanel Powerkey when startup receiver,the fontpanel will display as the picture 1

- Pictrue 1

- Press fontpanel ch+/ch- button switch the fontpanel display to picture 2.

- Picture 2

- Press Fontpanel OK button it will automatic startup and download file from usb stick..

If after download receiver still can not bootup please use  2.NAND Flash Programming.

2、NAND Flash Programming

- Setup a local network,with server computer ip 192.168.0.5

- Get the factory full flash file from forum and extract it to your server computer disk D:[make sure the folder is D:\tftpd] then setup the tftp tool accroding step 2.

- Tftpd.exe Setting

- Open the tftpd.exe.

- A）Click Tftpd->Configure

- B) Click the "Borwse" button, select the "tftpd" folder.

- Note: The path of the "tftpd" folder can not be too long.

- C) Input the number same like the picture displayed "Number of Clients" field .

- D）Click the "OK" button，then back to the main interface as follow picture:

- Click "Tftpd->Start"，

- Now, you finish the setting of the "tftpd.exe". During the following progress, you need not to  close the tool until you finish the programming of all the STB.

- How to update the STB.

1.Connect internet cable to STB[make sure both STB and server pc are in same local network]Press Fontpanel Powerkey when startup receiver,the fontpanel will display as the picture 1.

Picture 1

- Press fontpanel ch+/ch- button switch the fontpanel display to picture 2.

- Picture 2

- Press Fontpanel OK button it will automatic startup get data from server pc and update.

- When update the STB will display on fontpanel like step below:

"      DHCP    ",

"  SET SERVER  ",

"  SET LOADADDR ",

"   TFTP UBOOT ",

"   ERASE UBOOT ",

"   WRITE UBOOT ",

"  TFTP KERNEL ",

" ERASE KERNEL ",

" WRITE KERNEL ",

"   TFTP ROOT  ",

"  ERASE ROOT  ",

"  WRITE ROOT  ",

"  TFTP APPFS  ",

"  ERASE APPFS ",

"  WRITE APPFS ",

"  ERASE DATA  ",

"  TFTP DATA  ",

"  WRITE DATA ",

"  ERASE USER   ",

"  TFTP USER  ",

"  WRITE USER ",

"  ERASE DIVX  ",

"    TFTP K1   ",

"   ERASE K1   ",

"   WRITE K1   ",

"    TFTP R1   ",

"   ERASE R1   ",

"   WRITE R1   ",

"  TFTP ANDR_K ",

" ERASE ANDR_K ",

" WRITE ANDR_K ",

"  TFTP ANDR_R ",

" ERASE ANDR_R ",

" WRITE ANDR_R ",

"  TFTP ANDR_A ",

" ERASE ANDR_A ",

" WRITE ANDR_A ",

" ERASE ANDR_D ",

" ERASE ANDR_P ",

"    RESET     ",

- After finish STB will automatic startup the linux system.